## Steps to designing your own circuit board

### Before starting the design

- Find your libraries. These will be *.lbr files and can be found in a variety of places like GitHub. Libraries consist of user designed modules, using them means you don’t have design each on board component yourself, making the whole process much simpler.

- They should be saved inside the libraries folder of your Eagle program (usually inside C:\Documents\EAGLE\libraries)

- If you’re working in a group project, then using Autodesk’s library.io will allow many users to easier share the libraries that hey find. SunrIde VENSA has a library.io that the avionics team shares.

### Now you can start a new project

- Open Eagle and you will be presented with the central control panel.

  - File -> New -> Project

- Then right click on this new project and select;

  - New -> Schematic

- The first thing you should do is include the libraries that you have found

  - Library -> Library Manager -> Available -> Select USE

- Then you need to place a boarder to place your schematic inside

  - Add -> frame -> A4L – LOC -> OK -> place -> ESC -> ESC

- Now bring in the parts that you need

  - Teensy – 3.6_Only_Holes

  - LED5MM	4 single colour LEDS

  - R		4 resistors

  - 163		2 ADXL accelerometers

  - 35701331	2 relay

  - PINHD-1X2	2 1x2 pinheads

  - 1N4004		2 diodes

- Now you can go ahead and do all the GND pins

  - Add Part -> search GND -> sparkfun GND

- Then add the 5V pins

  - Add Part -> search 5V -> sparkfun 5V

- Then add the 3.3V pins

  - Add Part -> search 3.3V -> sparkfun 3.3V

- The pin named 3V3(Max_250MillaA) also needs a 3.3V connection, but in series with a resistor to limit the current. V/I = R gives 13.2 Ohms, the closest spec resistor in 15 Ohms.

- Now you can label the resistor using the Value tool on the left hand side

- Select each of the LED resistors and add the value 220 Ohms

- Then select the resistor connected to 3V3(Max_250) and give it the value of 15 Ohms

- Now you can connect up your schematic using the Net tool

- I’ve left the ADXL accelerometers unconnected as one of them must be renamed to ADXL377 which has a slightly different pin layout

- So, using the Name tool add the name ADXL377 to on accelerometer and ADXL335 to the other

- Now you can wire up both the accelerometers

- The last thing to do is connect and name the relay outputs. To do this simply create net lines out of each side of the switches and add names.

  - VBIN to the side with 1 above it

  - VBOUT to the side with 7 above it

# And now your schematic is finished